How to use the Prusa 3D printer and Slicer

- https://prusaslicer.net/  Download Most recent slicer and install

- Open the slicer. See attached photo for steps for 3 - 7.

- Print settings: 0.1 detail or 0.15 mm quality are good settings but can change this

- Printer: Prusa -i3 Mk3s and Mk3s+

- Select Filament. Usually PLA

- Support Everywhere

- Infill Hight the infill more is the material used. Use high value e.g., 90% for final product and low 10% or 15% for test prints.

- File > Import the stl file

- Then Slice and then Export the Gcode and copy to SD card

- SD card in 3D printer > Put some Gluue in the 3D-bed > choose G-code and start print

NOTE: At step 9 you can see the time to print. Modify the print settings and infill if needed to get a lower print time or better quality (more print time).

NOTE: The prusa has option to load/unload/print from SD card.